# LoomSense System Architecture

Purpose

Describe the major components of LoomSense, their responsibilities, and how they interact. This document intentionally focuses on architecture rather than implementation.

## Architectural Principles

Architecture follows the business domain rather than the hardware.

Mobile application is the system of record for projects and weaving sessions.

ESP32 hardware is responsible only for detecting treadle events and reporting them.

Communication between hardware and app should be lightweight and event driven.

The weaver always remains in control through overrides and manual corrections.

System should continue operating without cloud connectivity.

## High-Level Architecture

Weaver
   │
   ▼
ESP32 + Hall Sensors
   │
   ▼
Communication Layer
(Bluetooth/Wi-Fi)
   │
   ▼
LoomSense Mobile App
   ├── Project Manager
   ├── Session Manager
   ├── Sensor Processor
   ├── Drawdown Engine
   ├── Statistics Engine
   └── Local Data Store

## ESP32 Controller

Responsibilities:
• Read hall-effect sensors attached to treadles.
• Determine which treadle is pressed.
• Transmit treadle events to the mobile application.
• Remain unaware of weaving patterns or project logic.

## Communication Layer

Responsibilities:
• Reliably transmit treadle events.
• Report device connection status.
• Reconnect automatically after temporary interruptions.

## LoomSense Mobile Application

Acts as the primary application and source of truth.

Responsibilities:
• Manage projects.
• Manage weaving sessions.
• Interpret treadle events.
• Compare expected and actual treadles.
• Display weaving progress.
• Generate drawdowns.
• Record statistics and history.

## Project Manager

Loads, creates and stores immutable project designs.
Provides access to threading, sections, tie-ups and treadling sequences.

## Session Manager

Creates and resumes weaving sessions.
Tracks current section, pick, repeat, elapsed time and completion status.

## Sensor Processor

Receives treadle events from the ESP32.
Matches events against the active section's treadling sequence.
Supports undo and manual override.

## Drawdown Engine

Generates drawdowns from:
• Threading
• Section Tie-up
• Section Treadling Sequence

## Statistics Engine

Calculates elapsed time, picks, repeats, accuracy and future performance metrics from completed weaving sessions.

## Local Data Store

Stores projects, weaving sessions and application settings locally so LoomSense functions without requiring internet access.

## Information Flow

Weaver presses a treadle.

Hall sensor detects the treadle.

ESP32 identifies the treadle and sends an event.

Mobile app receives the event.

Sensor Processor validates the expected treadle.

Session Manager advances project progress.

Statistics and history are updated.

User interface refreshes with the next expected treadle.

## Separation of Responsibilities

| Component | Responsibility |
| --- | --- |
| ESP32 | Detect physical treadle presses. |
| Communication | Transport events. |
| Mobile App | Business logic and user experience. |
| Projects | Immutable weaving designs. |
| Weaving Sessions | Execution history and progress. |

## Future Expansion

Cloud synchronization

Multiple loom profiles

Advanced drawdown visualization

Performance analytics

Import/export of weaving drafts

Firmware updates over the air